Table S1. Index sets, notation, spatial layers, and model roles

Note. This table defines the notation, spatial layers, and computational roles used throughout the model. Parameter provenance is summarized in Table S9.

| Name / definition | Symbol or set | Role in the implemented model |
| --- | --- | --- |
| Time index | T={2012,…,2030}, t∈T | Annual time step. Historical reconstruction covers 2012--2023, the strict terminal holdout is 2022--2023 after training through 2021, and projection covers 2024--2030. |
| Scenario index | S={SC0,…,SC11}, s∈S | SC0 is the continuation baseline; SC1--SC7 are atomic levers; SC8--SC11 are integrated packages. |
| Region / spatial layer | R={C,R12,TFP}, r∈R | C denotes Tehran city-wide reporting, R12 denotes Region 12, and TFP denotes the traffic-plan exposure layer used for access-sensitive trips and revenue attribution. |
| Mode index | m∈M, Mr⊆M | M contains motorcycle, private car, taxi, bus, metro, and other. Bicycle is treated as a derived component within Other. BRT is treated as a structural submode within bus. |
| Public-transport mode subset | MPT={bus,metro} | Used for public-transport modal share, fare support, OPEX, subsidy, and farebox-recovery indicators. |
| Road-mode subset | Mroad={motorcycle,car,taxi,bus,other} | Used for road loading, PCE weighting, travel-time loss, and local tailpipe-emissions accounting. |
| Fuel or carrier index | c∈Cm, CmICE⊆Cm | Carrier or technology path compatible with mode m, including gasoline, diesel, CNG, and electricity. |
| Pollutant index | p∈P={CO2,NOx,PM2.5,SO2} | P contains CO2, NOx, PM2.5, and SO2, with CO2 used for climate accounting and local pollutants used for exposure and health-burden channels. |
| Agent index | i∈{1,…,n} | Indexes the 706 computational agents. By structural type, the model contains 237 inputs, 441 expression agents, 16 stocks, and 12 fixed ML surrogates; functional roles are reported below. |
| Agent type | typei∈{IN,EX,ST,ML} | Structural type: exogenous input (IN), expression (EX), stock (ST), or fixed ML surrogate (ML). Expression agents are assigned either SD or ABM functional roles. |
| Committed system state | Xt,s=x1,t,s,…,xn,t,s⊤ | Committed annual state used by all shared-state components after each simulation step. |
| Exogenous driver vector | Ut,s | Observed exogenous drivers are replayed in hindcast and projected through declared schedules; they do not overwrite endogenous states. |
| Scenario-overlay vector | Pt,s | Active scenario overlay for policy-sensitive variables and exogenous projection schedules in scenario s. |
| Projection window | Tproj={2024,…,2030}, e=2030 | Projection window used for KPI summaries and decision scoring; e denotes the terminal projection year (2030). |
| Policy-scenario set | Spol={SC1,…,SC11} | Non-baseline scenario set used for max-absolute normalization of oriented cumulative effects. |
| Core decision domains | D={dtr,denv,dsoc,decon} | Four outcome domains used only in the preference-conditioned decision score: Transportation, Environmental, Social, and Economic. These are distinct from the calibration target domains. |
| Retained KPI set for domain d | Kd, d∈D | Three representative KPI subfamilies are retained per domain; highly collinear indicators remain in raw outputs and sensitivity checks. |
| Orientation coefficients | ok∈{-1,+1}, k∈K; ojimpl∈{-1,+1}, j∈Jimpl | oₖ maps each retained outcome KPI to its preferred direction before decision normalization; oⱼⁱᵐᵖˡ does the same for implementation-screen indicators. Figure 9 remains signed and unoriented. |
| Reporting geography subset | Rout={C,R12}⊂R | Geographies used for KPI reporting and decision scoring; the TFP layer remains an exposure and revenue sublayer rather than a separate outcome geography. |
| Health exposure species / source mapping | Ph={PM2.5,NO2}, χ:Ph→P | Exposure-response species used in city health accounting; χ maps PM₂.₅ to PM₂.₅ and NO₂ to NOₓ in the source emissions inventory. |
| Global parameter collection | Θ={θSD,θABM,θML} | Global parameter collection appearing in main-text Eq. 1; it is not the search space of the local recovery analysis. |
| ML lookback set | q∈Qm,r | Admissible lag order/set used to construct the mode- and region-specific ML feature history in Eq. 2. |
| Calibration target-domain index and weights | dc∈Dcal, wdccal≥0, dc∈Dcal​wdccal=1 | Seven calibration target domains used only in Eq. 12: Tehran modal shares, R12 modal shares, speed, travel activity, energy/environment, annual vehicle distance, and operational shares. The calibration-domain index and weights are distinct from the decision-domain index and preference weights used after simulation. |
| Local recovery parameter space | V⊂R4 | Admissible four-dimensional scalar search space used by the local parameter-recovery analysis; distinct from the global model parameter collection Θ. |
| Asset / infrastructure index | a∈A | Infrastructure or procurement asset class used in network, CAPEX, and unit-cost relations. |
| KPI index | k∈K | Outcome-indicator index used in baseline-relative effects, normalization, and domain scores. |
| Trip-length class index | g∈G | Trip-length class used to map modes and spatial layers to representative trip lengths. |
| Validation-comparison index | v∈V | Indexes valid observed-simulated metric-year comparisons in pooled validation measures. |
| Implementation-indicator index | j∈Jimpl | Index for implementation-side feasibility indicators used in the separate implementation screen. |
| Policy-control channel index | jp∈Jpol | Indexes scenario-overlay channels in the policy schedule table. |
| Functional role assignment | Input / SD / ABM / ML | The same 706 agents are functionally classified as 237 inputs, 400 SD roles, 57 ABM mediation roles, and 12 ML modal-prior roles. The SD group comprises 384 aggregate expressions and 16 stocks; the ABM group comprises 57 mediation expressions. |

Structural and functional counts refer to the same 706-agent specification. Every retained agent contributes to simulation, lag construction, or reported outputs.