Table S6. Scenario overlays and operator schedules over 2024–2030

Note. Full, medium, light, and strong denote fixed package intensities rather than optimized values. Schedules are rounded for presentation; the simulations use the declared annual settings.

| Scenario | Primary operator channel | Policy schedule, 2024–2030 |
| --- | --- | --- |
| SC0 | Continuation baseline | No additional policy overlay; all paths follow the continuation reference. |
| SC1 | Activity moderation | Effective telework rate ρttw: 2024–2025 = 0; 2026–2027 = 0.08; 2028–2029 = 0.12; 2030 = 0.15. Commute-management multiplier μtcommute: 2024–2025 = 1; 2026–2027 = 1.12; 2028–2029 = 1.20; 2030 = 1.26. |
| SC2 | Managed core access pricing | Traffic-plan fee multiplier μtfee: 2024–2025 = 1; 2026–2027 = 1.15; 2028 = 1.25; 2029 = 1.30; 2030 = 1.35. |
| SC3 | Parking and curb management | Region 12 parking-supply multiplier μR12,tpark: 2024–2025 = 1; 2026–2027 = 0.94; 2028–2029 = 0.88; 2030 = 0.82. |
| SC4 | PT affordability support | Bus fare-growth rate gtfare,bus: 2024–2027 = 0.24; 2028–2030 = 0.25. Metro fare-growth rate gtfare,metro: 2024–2027 = 0.21; 2028–2030 = 0.22. |
| SC5 | PT service, access, and reliability | Bus free-flow speed vtbus,free: 2024–2025 = 40.0 km/h; 2026 = 42.0; 2027 = 43.5; 2028–2030 = 45.0. Metro operating speed vtmetro: 2024–2025 = 42.0 km/h; 2026 = 43.0; 2027 = 44.0; 2028–2030 = 45.0. The service overlay also includes staged bus, BRT, and metro network and procurement support. |
| SC6 | Local pollutant cleanup | Motorcycle emission factors decline as follows: EFmot,tPM2.5 = 0.03 in 2024, 0.021 in 2025–2027, 0.015 in 2028–2029, and 0.012 g/km in 2030; EFmot,tNOx = 0.40 in 2024, 0.32 in 2025–2027, 0.24 in 2028–2029, and 0.20 g/km in 2030; EFmot,tSO2 = 0.02 in 2024, 0.016 in 2025–2027, 0.012 in 2028–2029, and 0.01 g/km in 2030. Bus and taxi pollutant factors decline along the corresponding cleanup paths; bus speed receives a limited operational co-benefit up to 41.4 km/h by 2030. |
| SC7 | Clean-fleet transition | Taxi EV share qtaxi,tEV: 2024 = 0.04; 2025 = 0.06; 2026 = 0.12; 2027 = 0.16; 2028 = 0.28; 2029 = 0.36; 2030 = 0.44. Bus EV share qbus,tEV: 2024 = 0.10; 2025 = 0.12; 2026 = 0.16; 2027 = 0.20; 2028 = 0.26; 2029 = 0.32; 2030 = 0.40. |
| SC8 | Balanced package | Composition: SC1 full + SC3 light + SC4 full + SC6 medium. Region 12 parking-supply multiplier μR12,tpark: 2024–2025 = 1; 2026–2027 = 0.976; 2028–2029 = 0.952; 2030 = 0.928. Cleanup co-benefit for bus free-flow speed: 2024–2025 = 40.0 km/h; 2026 = 40.33; 2027 = 40.46; 2028 = 40.65; 2029 = 40.78; 2030 = 40.91. Fare-support paths follow SC4; local pollutant factors follow the medium cleanup path. |
| SC9 | Access-led package | Composition: SC2 full + SC3 light + SC4 medium + SC5 medium + SC6 light. The access-charge path follows SC2. Region 12 parking-supply multiplier μR12,tpark: 2024–2025 = 1; 2026–2027 = 0.979; 2028–2029 = 0.958; 2030 = 0.937. Bus fare-growth rate: 2024–2027 = 0.281; 2028–2030 = 0.286. Metro fare-growth rate: 2024–2027 = 0.242; 2028–2030 = 0.247. Bus free-flow speed: 2024–2025 = 39.34 km/h; 2026 = 40.54; 2027 = 41.44; 2028–2030 = 42.34. Metro speed: 2024–2025 = 41.2 km/h; 2026 = 41.8; 2027 = 42.4; 2028–2030 = 43.0. Pollutant factors follow the light cleanup path. |
| SC10 | PT-first clean package | Composition: SC4 full + SC5 full + SC6 strong + SC7 medium + SC3 light. Region 12 parking-supply multiplier μR12,tpark: 2024–2025 = 1; 2026–2027 = 0.985; 2028–2029 = 0.970; 2030 = 0.955. SC4 fare support and SC5 service paths are used at full intensity. Terminal cleanup values include EFmot,2030PM2.5=0.0156 g/km, EFmot,2030NOx=0.24 g/km, and EFmot,2030SO2=0.012 g/km. Medium EV package intensity gives qtaxi,tEV = 0.02, 0.026, 0.044, 0.056, 0.092, 0.116, 0.14 for 2024–2030 and qbus,tEV = 0.05, 0.056, 0.068, 0.08, 0.098, 0.116, 0.14 for 2024–2030. |
| SC11 | Broad package | Composition: SC1 medium + SC2 medium + SC3 medium + SC5 full + SC6 medium + SC7 medium. Effective telework rate ρttw: 2024–2025 = 0; 2026–2027 = 0.052; 2028–2029 = 0.078; 2030 = 0.0975. Traffic-plan fee multiplier μtfee: 2024–2025 = 1; 2026–2027 = 1.10; 2028 = 1.16; 2029 = 1.20; 2030 = 1.23. Region 12 parking-supply multiplier μR12,tpark: 2024–2025 = 1; 2026–2027 = 0.964; 2028–2029 = 0.928; 2030 = 0.892. PT service paths follow the full SC5 operator; cleanup and clean-fleet transition use medium package intensities. Medium EV package intensity gives qtaxi,tEV = 0.02, 0.026, 0.044, 0.056, 0.092, 0.116, 0.14 for 2024–2030 and qbus,tEV = 0.05, 0.056, 0.068, 0.08, 0.098, 0.116, 0.14 for 2024–2030. |